06601030305800

9[...................................................]001

2600/7800 DEVELOPMENT KIT

CARE AND FEEDING INSTRUCTIONS

A full, complete 2600/7800 development system includes the following:

HARDWARE:

520ST or 1040ST COMPUTER or 520STFM (w/ built-in drive)

SH204 HARD DISK

SF354 or SF314 FLOPPY DISK (not incl. w/ 520STFM or 1040ST)

SM124 or SC1224 MONITOR

7800 GAME MACHINE - O.S. ROM REPLACED BY TRANSFER ROM ASSEMBLY

CABLE

RAM CARTRIDGE

TELEVISION (notsuppliedbyAtari)

EPROM CARTRIDGE (FOR APPROPRIATE GAME SYSTEM)

SOFTWARE:

DEVELOPMENT KIT DISK:

40COL.S

ASM.BAT

BATCH.TTP

CONV65.PRG

DOWNLOAD.PRG

K.TTP

MAC.PRG (SERIALIZED)

MARIA.S

ME.TTP

WAIT.PRG

Also included:  *

ALN.PRG

ATARI.S

DOINDEX.PRG

INSTMAC.PRG

M.PRG

(various "readme" files)

(MADMAC "EXAMPLES" folder)

HARD DISK BOOT DISK (COMES WITH HARD DISK DRIVE)

DOCUMENTATION FOR 2600 DEVELOPMENT:

ASSEMBLER MANUAL

EDITOR MANUAL

STELLA (2600) PROGRAMMER'S GUIDE

2600 GAME STANDARDS AND PROCEDURES

7800 SUPER CART SPEC

"USING THE SUPERCART... FOR DEVELOPMENT"

SARA SPEC

DOCUMENTATION FOR 7800 DEVELOPMENT:

ASSEMBLER MANUAL

EDITOR MANUAL

7800 PROGRAMMING GUIDE

MARIA SPEC

MARIA DIGEST (SHORT AND SUPERSHORT VERSIONS)

STELLA (2600) PROGRAMMER'S GUIDE

7800 GAME STANDARDS AND PROCEDURES

7800 SUPER CART SPEC

"USING THE SUPERCART... FOR DEVELOPMENT"

Set up the ST computer per its instructions.

If your 7800 has not been equipped with transfer ROM assembly,

you must install it according to the following instructions:

To open up the 7800, unscrew the bottom.  Lift the shielded

PCB from the plastic housing.  Remove RF shield by twisting

tabs.  Refer to attached sketch of PCB for position of OS

rom.  Remove OS rom in easiest manner.  If you have older

7800, simply pull rom from its socket.  If you have newer

7800, clip the rom pins and remove solder from PCB holes.

Install transfer rom, noting correct pin 1 position on

attached sketch.  Put 7800 back together if desired and/or

possible.  Note that it will be easier to remember which

buttons are Pause, Reset, and Select if plastic housing is

in place.

Connect 7800 to power supply, attach 7800 to TV set.

Attach custom transfer cable to printer port of ST and to the

joystick ports of 7800.  You will probably want to make a

switchbox for the cable, so that you won't have to

constantly remove the cable to plug in a joystick, and vice

versa.

If your supercart RAM card has not been equipped with a Write-Enable

switch, you will need to do that.  See supercart schematic

(if you do not have one and need one, call Tom Sloper).

Put as much RAM as you will need on the 1-meg ROM emulator.

It is set up for (4) 256K bits RAM chips.

Put the RAM card in the cartridge port (chips to the rear).

And off you go!

The 6502 cross-assembler is proprietary.  Your serial number is

registered in your name.  Pirating is a no-no!

If there are any questions regarding the use of the software or

hardware, call John Feagans at Atari.  (408) 745-4923

With any other problems, call Tom Sloper.  (408) 745-4874

tas 6/17/87